Question Bank

On

Computer Engineering Drawing (22CED13)

I SEM BE (ODD SEMESTER)

2022-23

CHAPTER 1

PROJECTIONS OF POINTS

- Draw the projections of the following Points on the same XY line, keeping convenient distance between each projector. Name the Quadrants in which they lie.

A - 30mm above HP & 35mm in front of VP.

B - 35mm above HP & 40mm behind VP.

C - 40mm above HP & on VP

D - 35 mm below HP & 30 mm in front of VP

- Draw the projections of the following points on the same XY line, keeping convenient distance between the projectors. Also state the quadrants in which they lie.

A – 30 mm below HP & 25 mm behind VP.

B – 35 mm below HP & 30 mm in front of VP.

C – on HP &30 mm in front of VP.

D –on HP & 35 mm behind of VP.

- Two Points R and S are on HP. The point R is 35 mm in front of VP, while S is 50 mm behind VP. The line joining their top views makes an angle of 400 with XY. Find the horizontal distance between the two projectors.

- A point ‘M’ is 30 mm in front of VP and 20 mm above HP. Another point N in 15mm behind VP & 25 mm below HP. The horizontal distance between the points parallel to XY line is 50 mm. Draw the projections of the points M & N and join front and top views. Draw the right side view for the point N only.

- A point A is 20 mm above HP and is in the first quadrant. Its shortest distance from the XY line is 40 mm, draw the projections. Determine its distance from VP.

- Draw the projections of a point A lying 30 mm above HP and in first quadrant, if its shortest distance from the line of intersection of HP and VP is 50 mm. Also find the distance of the point from VP.

CHAPTER 2

PROJECTIONS OF LINES

      - A line AB 80 mm long has its end A 20 mm above the HP and 30 mm in front of VP. It is inclined at 30º to HP and 45 to VP. Draw the projections of the line and find apparent lengths and apparent inclinations.

- A line AB has its end A 20 mm above the HP and 30 mm in front of the VP. The other end B is 60 mm above the HP. and 45 mm in front of VP. The distance between end projectors is 70 mm. Draw its projections. Determine the true length and apparent inclinations.

- The top view PQ of a straight line is 70 mm and makes an angle of 600 with XY line. The end Q is 10 mm in front of VP and 30 mm above the HP. The difference between the distances of P and Q above the HP is 45 mm. draw the projections. Determine its true length and true inclinations with HP and VP.

- A line has its end A 10 mm above HP and 15 mm in front of VP. The end B is 55 mm above HP and line is inclined at 30o to HP 35o to VP. The distance end projectors is 50mm. Draw the projections of the line. Determine the true length of the line and its inclination with VP.

- The top view of a 75 mm long line AB measures 65 mm, while the front view is 50 mm. Its one end A is in the HP and 12 mm in front of the VP. Draw the projections of AB and determines its inclinations with the HP and the VP.

- A point P is 40 mm above HP and 20 mm in front of VP another point Q is 20 mm above HP and 50 mm in front of VP. The top view of line PQ is inclined at 30o to XY. Draw the projections.

CHAPTER 3

PROJECTIONS OF PLANE SURFACES

- An equilateral triangular lamina of 25 mm side lies with one of its edges on HP such the surface of the lamina is inclined to HP at 60º. The edge on which it rests is inclined to VP at 60. Draw the projections.

- A Triangular lamina of 25 mm sides on one of its corners on VP such the median passing through the corner on which it rests is inclined at 30º to HP and 45º to VP. Draw its projections.

- A 30º- 60º setsquare of 60 mm longest side is so kept such that the longest side is in HP, making an angle of 30º with VP. The set square itself is inclined at 45º to HP. Draw the projections of the setsquare.

- A square plate of 40 mm sides rests on HP such that one of the diagonals is inclined at 30º to HP and 45º to VP. Draw its projections.

- A rectangular plate of negligible thickness of size 35mm x 20mm has one of its shorter edges in VP with that edge inclined at 40º to HP. Draw the projections if the top view if its front view is a square of side 20mm.

- A pentagonal lamina of edges 25mm is resting on HP with one of its sides such that the surface makes an angle of 60º with HP. The edge on which it rests is inclined at 45º to VP. Draw its projections.

- A pentagonal lamina having edges 25mm is placed on one of its corners on VP such that the surface makes an angle 30º with VP perpendicular bisector of the edge passing through the corner on which the lamina rests is inclined at 45º to HP. Draw the top and front views of the lamina.

- A hexagonal lamina of sides 25mm rests on one of its sides on HP. The lamina makes 45º to HP and the side on which it rests makes 30º to VP. Draw its projections.

- A circular lamina of 50mm diameter rests on HP such that one of its diameters is inclined at 30º to VP and 45º to HP. Draw its top and front views in this position.

CHAPTER 4

PROJECTIONS OF SOLIDS

- A square prism 35mm sides of base and 60mm axis length rests on HP on one of its edges of the base which is inclined to VP at 30º. Draw the projections of the prism when the axis is inclined to HP at 45º.

- A square prism 35mm sides of base and 60mm axis length rests on HP on one of its corners of the base such that the two base edges containing the corner on which it rests make equal inclinations with HP. Draw the projections of the prism when the axis of the prism is inclined to HP at 40º and to VP at 30º.

- A pentagonal prism 25mm sides of base and 60mm axis length rests on HP on one of its edges of the base which is inclined to VP at 30º. Draw the projections of the prism when the axis is inclined to HP at 40º.

- A pentagonal prism 25mm sides of base and 60mm axis length rests on HP on one of its corners of the base such that the two base edges containing the corner on which it rests make equal inclinations with HP. Draw the projections of the prism when the axis of the prism is inclined to HP at 40º and to VP at 30º.

- A hexagonal prism 25 mm sides of base and 50mm axis length rests on HP on one of its edges of base. Draw the projections of the prism when the axis is inclined to HP at 45º and VP at 30º.

- A hexagonal prism 25 mm sides of base and 50mm axis length rests on HP on one of its corners of the base such that the two base edges containing the corner on which it rests make equal inclinations with HP. Draw the projections of the prism when the axis of the prism is inclined to HP at 40º and appears to be inclined to VP at 45º.

- A square pyramid 35 mm sides of base and 65 mm axis length rests on HP on one of its edges of the base which is inclined to VP at 30º. Draw the projections of the pyramid when the axis is inclined to HP at 45º.

- A square pyramid 35 mm sides of base and 60 mm axis length rests on HP on one of its corners of the base such that the two base edges containing the corner on which it rests make equal inclinations with HP. Draw the projections of the pyramid when the axis of the pyramid is inclined to HP at 40º and to VP at 30º.

- A tetrahedron of 55 mm sides rests on one of its corners such that an edge containing that corner is inclined to HP at 50º and VP at 30º. Draw its projections.

- A cone of 50mm base diameter and 60 mm axis length rests on HP on one of its generators. Draw its projections when the axis is inclined to VP at 30º.

CHAPTER 5

DEVELOPMENT OF LATERAL SURFACES OF SOLIDS

- A square prism of base side 40mm and axis length 65mm is resting on HP on its base with all the vertical faces being equally inclined to VP. It is cut by an inclined plane 60º to HP and perpendicular to VP and is passing through a point on the axis at a distance 15mm from the top face. Draw the development of the lower portion of the prism.

- A rectangular prism of base 40mmX25mm and height 65mm rests on HP on its base with the longer base side inclined at 300 to VP. It is cut by a plane inclined at 400 to HP perpendicular to VP cuts the axis at its mid height. Draw the development of the remaining portion of the prism.

- A pentagonal prism of base sides 30 mm and axis length 60 mm rests with its base on HP and an edge of the base inclined at 450 to VP. It is cut by a plane perpendicular to VP, inclined at 400 to HP and passing through a point on the axis, at a distance of 30mm from the base. Develop the remaining surfaces of the truncated prism.

- A square pyramid of side of base 45mm, altitude 70mm is resting with its base on HP with two sides of the base parallel to VP. The pyramid is cut by a section plane which is perpendicular to the VP and inclined at 40º to the HP. The cutting plane bisects the axis of the pyramid. Obtain the development of the lateral surfaces the truncated pyramid.

- A pentagonal pyramid 30mm edge of base and 50mm height rests vertically with one of its base edges parallel to VP and nearer to it. It is cut as shown in following figure. Draw the development of the lateral surfaces of the upper portion of the pyramid.

- Develop the lateral surface of the cylinder of 40mm diameter and height 60mm which is cut in the following way.

- Draw the development of the lateral surface of the cone, whose front view is as shown in the following figure.

CHAPTER 6

ISOMETRIC PROJECTION

      - A sphere of diameter 50mm rests centrally on top of a cube of sides 50mm.Draw the isometric projections of the combination of solids.

- The frustum of a square pyramid of sides 40mm, top face side 20mm and height 60mm rest on the top of a square block of side 60mm and height 20mm. The base edge of the pyramid is parallel to the top edges of the square block. Draw the isometric projection of combination of the solids.

- A cone of base diameter 30mm and height 40mm rests centrally over a cube of side 50mm.  Draw the isometric projection of the combination of solids.

- A hemisphere of diameter 50mm is centrally resting on top of a square prism of base side 60mm and height 30mm such that the curved surface of hemisphere is touching the top face of the prism. Draw its isometric projections.

- A pentagonal pyramid base side 25mm and height 65mm is placed centrally on a rectangular slab 100mm x 60mm and 20mm thick. Draw the isometric projections of the combination.

- A sphere diameter 40mm is placed centrally on the flat face of a hemisphere diameter 60mm. Draw the isometric projections of the combination.